CCNA REVIEW NOTES

**DISCLAIMER: This is not an official CCNA study guide. All topics listed are in reference to the CCNA Exam Topics list posted to Cisco.com. This document assumes the reader has already studied the foundational concepts and should be used for review only.

IP Addressing

IPv4 / IPv6  Multicast Addresses:

| Address For | IPv6 Address | IPv4 Address | Note |
| --- | --- | --- | --- |
| All Nodes | FF02::1 | 224.0.0.1 | NDP RA messages |
| All Routers | FF02::2 | 224.0.0.2 | HSRP V1, NDP RS messages |
| All OSPF Routers | FF02::5 | 224.0.0.5 |  |
| ALL OSPF DR/BDRs | FF02::6 | 224.0.0.6 |  |
| ALL RIP Routers | FF02::9 | 224.0.0.9 |  |
| ALL EIGRP Routers | FF02::A | 224.0.0.10 |  |
| VRRP |  | 224.0.0.18 |  |
| GLBP |  | 224.0.0.102 | HSRP V2 |

IPv4 Public Classes

| Class A | 0.0.0.0 | 127.255.255.255 |
| --- | --- | --- |
| Class B | 128.0.0.0 | 191.255.255.255 |
| Class C | 192.0.0.0 | 223.255.255.255 |
| Loopback | 127.0.0.0 | 127.255.255.255 |

IPv4 Private Address Ranges

| 10.0.0.0 | 10.255.255.255 /8 |
| --- | --- |
| 172.16.0.0 | 172.31.255.255 /12 |
| 192.168.0.0 | 192.168.255.255 /16 |

IPv6 Address Types

| Name | Range | Note |
| --- | --- | --- |
| Global Unicast | Any that’s not reserved | Public, globally unique |
| Unique Local | FC00::/7 (Starts with FD) | Like an IPv4 private address |
| Link-Local | FE80::/10 | Auto generated with EUI-64 |
| Multicast | FF00::/8 (Usually FF02) | One-to-many communication |
| Anycast | Any that's not reserved | When config., specify anycast |

IPv6 Multicast Scopes

| Interface-local | FF01 |
| --- | --- |
| Link-Local | FF02 |
| Site-local | FF05 |
| Organization-local | FF08 |
| Global | FF0E |

EUI-64 Generation Steps

| 1.) | Divide MAC address in half |
| --- | --- |
| 2.) | Insert FFFE in the middle |
| 3.) | Invert 7th bit (2nd hexadecimal in the MAC address) Ex: 8 -> A |

Solicited-node Multicast Address

ff02::1:ff [last 6 hex digits of unicast address]

Example: 2001:db8::1:f2a:4fff:fea3:91b1 → ff02::1:ffa3:91b1

NDP Message Types

| RS (Router Solicitation) | ICMPv6 Type 133 | FF02::2, SLAAC messages |
| --- | --- | --- |
| RA (Router Advertisement) | ICMPv6 Type 134 | FF02::1, SLAAC messages |
| NS (Neighbor Solicitation) | ICMPv6 Type 135 | DAD messages |
| NA (Neighbor Advertisement) | ICMPv6 Type 136 | DAD messages |

MAC Addresses

| Broadcast | ffff.ffff.ffff |
| --- | --- |
| HSRP V1 Virtual Format | 0000.0c07.acxx (xx = group number) |
| HSRP V2 Virtual Format | 0000.0c9f.fxxx (xxx = group number) |
| VRRP Virtual Format | 0000.5e00.01xx (xx = group number) |
| GLBP Virtual Format | 0007.b400.xxyy (xx = group, yy = router) |
| Standard STP BPDU’s Destination MAC | 0180.c200.0000 |
| PVST+ BPDU’s Destination MAC | 0100.0ccc.cccd |
| LLDP Destination MAC | 0180.c200.000e |
| CDP Destination MAC | 0100.0ccc.cccc |

Cable Topics

Copper Ethernet Cable Standards

| 10 Mbps | Ethernet | 802.3i | 10 Base-T | 100m |
| --- | --- | --- | --- | --- |
| 100 Mbps | Fast Ethernet | 802.3u | 100 Base-T | 100m |
| 1 Gbps | Gigabit Ethernet | 802.3ab | 1000 Base-T | 100m |
| 10 Gbps | 10 Gig Ethernet | 802.3an | 10G Base-T | 100m |

Pin Pairs

| Device | Transmits on | Received on |
| --- | --- | --- |
| Firewall | 1 and 2 | 3 and 6 |
| Router | 1 and 2 | 3 and 6 |
| PC | 1 and 2 | 3 and 6 |
| Switch | 3 and 6 | 1 and 2 |

Fiber Cable Standards

| 1000 Base-LX | 802.3z | 1 Gbps | Multi or Single-Mode | 550m (MM) 5 KM (SM) |
| --- | --- | --- | --- | --- |
| 10G Base-SR | 802.3ae | 10 Gbps | Multimode | 400 m |
| 10G Base-LR | 802.3ae | 10 Gbps | Single-mode | 10 km |
| 10G Base-ER | 802.3ae | 10 Gbps | Single-mode | 30 km |

Serial Standards (WAN)

| T1 (slowest) | 1.544 Mbps | North America / Japan |
| --- | --- | --- |
| T2 | 6.312 Mbps | North America / Japan |
| T3 (fastest) | 44.736 Mbps | North America / Japan |
| E1 | 2.048 Mbps | Europe |
| E2 | 8.448 Mbps | Europe |
| E3 | 34.368 Mbps | Europe |

Cable Terminators

| Ethernet | RJ45 |
| --- | --- |
| Fiber | ST |
| Point-to-Point Serial | DB-9 |
| Coaxial | BNC |
| POTS | RJ-11 |

Duplex Errors

| Symptoms | Runts, FCS and CRC errors, Alignment errors, Late collisions |
| --- | --- |

If Duplex Autonegotiation is Off on One Device

| Speed | Other device will try to sense speed, if it can't; it will operate at slowest supported speed |
| --- | --- |
| Duplex | 10 or 100 Mbps = Half duplex 1000Mbps (1Gbps) or More = Full duplex |

Headers / Segments

Ethernet Header

IPv4 Header

IPv6 Header

UDP Segment

TCP Segment

Port numbers

| TCP | UDP | TCP & UDP |
| --- | --- | --- |
| FTP Data (20) | DHCP Server (67) | DNS (53) (Usually UDP) |
| FTP Control (21) | DHCP Client (68) | DNS uses TCP > 512 bytes |
| SSH (22) | TFTP (69) |  |
| Telnet (23) | NTP (123) |  |
| SMTP (25) | SNMP Agent (161) |  |
| TACACS+ (49) | SNMP Manager (162) |  |
| HTTP (80) | Syslog (514) |  |
| POP3 (110) | RADIUS (1812) |  |
| HTTPS (443) | CAPWAP Control (5246) |  |
|  | CAPWAP Data (5247) |  |

Layer 4 Port Ranges

| Well-Known Ports | 0 - 1023 |
| --- | --- |
| Registered Ports | 1024 - 49151 |
| Ephemeral Ports | 49152 - 65535 |

IPv4 Header Protocol Numbers

| ICMP | 1 |
| --- | --- |
| TCP | 6 |
| UDP | 17 |
| EIGRP | 88 |
| OSPF | 89 |

Administrative Distances

| Route Protocol / Type | AD |
| --- | --- |
| Directly Connected | 0 |
| Static | 1 |
| External BGP (eBGP) | 20 |
| EIGRP | 90 |
| IGRP | 100 |
| OSPF | 110 |
| IS-IS | 115 |
| RIP | 120 |
| EIGRP (external) | 170 |
| Internal BGP (iBGP) | 200 |
| Unusable Route | 255 |

Random Protocol Topics

STP/RSTP Root Bridge Election

- Lowest Bridge Priority

- Tiebreaker: Lowest MAC Address

STP/RSTP Root Port Election

- Lowest Root Cost (bandwidth)

- Lowest Bridge ID (Bridge Priority + MAC Address)

- Lowest Neighbor Port ID

STP Standards

| Classic STP | 802.1D |
| --- | --- |
| Multiple STP | 802.1s |
| PVST+ | Cisco upgrade to 802.1D |
| RSTP | 802.1w |
| Rapid PVST+ | Cisco upgrade to 802.1w |

RSTP Port Roles

| Root | To Root Bridge (forwarding) |
| --- | --- |
| Designated | From Root Bridge and on Some Other Bridges (forwarding) |
| Alternate | Backup to Root Port (discarding) superior BPDU from other switch |
| Backup | Usually for Hubs (discarding) superior BPDU from same switch |

Etherchannel

| PAgP (Cisco Proprietary) | LACP (802.3ad Standard) |
| --- | --- |
| Channel-protocol pagp | Channel-protocol lacp |
| Desirable, Auto | Active, Passive |

Etherchannel Commands

| Where / Type of Command | Term used | Examples |
| --- | --- | --- |
| Global configuration (config)# | port-channel | Port-channel load-balance Interface port-channel <num> |
| Interface configuration (config-if)# | channel-group | Channel-group <num> mode Channel-protocol <protocol> |
| Show commands # | etherchannel | Show etherchannel summary |

Layer 3 Etherchannel Configuration

| - Enable routing (L3 switches) | (config)# ip routing |
| --- | --- |
| - Make SVI, assign to channel-group | (config-if)# no switchport (config-if)# channel-group 1 mode <mode> |
| - Enter config. For that port-channel | (config)# int port-channel 1 |
| - Assign IP and enable | (config-if)# ip address 192.168.1.1 255.255.255.0 (config-if)# no shut |

CDP / LLDP Info

| Name | Timer | Holdtime | Note |
| --- | --- | --- | --- |
| CDP (Cisco) | 60 sec | 180 sec | Shows VTP in detail command, enabled by default |
| LLDP (802.1ab) | 30 sec | 120 sec | Tx and Rx on interfaces separately enabled, default re-initialization timer is 2 seconds |

CDP Show Commands

| Show cdp neighbors | Device ID, Local Int, Holdtime, Capabilities, Product Number, Neighbor Port ID |
| --- | --- |
| Show cdp neighbors detail | Everything in normal command + IP Address, Software Version, Native VLAN, VTP Domain |
| Show cdp entry <name> | Like Detail, but for a specific device in the network |

OSPF DR/BDR Election

- Highest Priority (1-255), Second Highest becomes BDR (default is 1)

- Tiebreaker: Highest Router ID

OSPF Router ID Selection

- Manually Configured Router ID

- Highest Loopback Interface IP Address

- Highest Physical Interface IP Address

OSPF Neighbor States

| Down | No Neighbors yet |
| --- | --- |
| Init | Hello received, own Router ID not in packet |
| 2-Way | Received Hello with own Router ID, DR and BDR elected |
| Exstart | Choose Master/Slave, Higher Router ID becomes master |
| Exchange | Exchange DBDs |
| Loading | Send LSR and LSU to update their LSDBs |
| Full | Adjacency, DROthers only form Full with DR and BDR |

OSPF Network Types

| Broadcast | Point-to-Point | Nonbroadcast |
| --- | --- | --- |
| DR and BDR elected | No DR or BDR | DR and BDR elected |
| Multicast | Multicast | Unicast |
| Ethernet and FDDI Interfaces | HDLC and PPP Interfaces | Frame Relay and X.25 Interfaces |
| Hello 10, Dead 40 | Hello 10, Dead 40 | Hello 30, Dead 120 |
| Dynamic Adjacencies | Dynamic Adjacencies | Manual Adjacencies (neighbor command) |

OSPF LSA Types

| Type | Note |
| --- | --- |
| Type 1 (Router LSA) | Generated by every router, lists Router ID, networks attached |
| Type 2 (Network LSA) | Generated by the DR, lists routers attached to the multi-access network |
| Type 5 (AS-External LSA) | Generated by ASBR, advertises routes to destinations outside the AS |

Requirements for OSPF Adjacency

Area Number must match

Interfaces must be in same subnet

OSPF process must not be shutdown

OSPF Router IDs must be unique

Hello and Dead timers must match

Authentication Settings must match

Other Requirements for OSPF Adjacency

| IP MTU Settings must match | Become neighbors, but won’t go to Full state |
| --- | --- |
| OSPF Network Type must match | Prevents proper adjacency, won’t learn OSPF routes |

Random OSPF Configuration Commands

| Configure Cost of an Interface | (config-if)# ip ospf cost <cost> |
| --- | --- |
| Configure the Reference Bandwidth | (config-router)# auto-cost reference-bandwidth <mbps> |
| Enable OSPF Directly on an Interface | (config-if)# ip ospf <process-id> area <area num> |
| Enable OSPF Authentication | (config-if)# ip ospf authentication |
| Configure an OSPF Password | (config-if)# ip ospf authentication-key <password> |
| Set the OSPF Network Type on an Int. | (config-if)# ip ospf network <type> |

Dynamic Routing Protocol Comparison

| OSPF | RIP | EIGRP |
| --- | --- | --- |
| Link-state | Distance Vector | Distance Vector |
| Router ospf [process-ID] | Router rip | Router eigrp [AS-number] |
| Network <ip> <wildcard> area <num> | Network <ip> | Network <ip> <wildcard> |
| Does not have auto-summary | Auto-summary enabled, default (should turn off) | Auto-summary enabled, default (should turn off) |
| Area must match, process-ID does not have to match between routers | n/a | AS-number must match between routers |
| Bandwidth (cost) metric | Hop count metric | Bandwidth (slowest link) + delay (all links) metric |
| Only ECMP load-balance | Only ECMP load-balance | Supports different cost load-balance (using feasible successor routes) |

EIGRP Terms

| Successor | Route with lowest metric to destination (best path) |
| --- | --- |
| Feasible Successor | Alternate route which meets the feasibility condition |
| Feasible Distance | The router’s own metric to the destination |
| Reported Distance | The neighbor’s metric value to the destination |

Feasibility Condition

A route is considered a Feasible Successor if its Reported Distance is lower than the Successor route’s Feasible Distance.

FHRP Protocols

| HSRP | Active, Standby (Cisco) (Load-Balance per VLAN) (ONE Virtual MAC) |
| --- | --- |
| VRRP | Master, Backup (Industry-standard) (Load-Balance per VLAN) |
| GLBP | AVG, AVF (Cisco) (Same VLAN Load-Balancing via AVFs) |

HSRP Active Router Election

- Highest Priority (Default 100)

- Highest IP Address

SNMP Messages

| Message Type | SNMP Message Name | Note |
| --- | --- | --- |
| Read | Get, GetNext, GetBulk | Sent by NMS |
| Write | Set | Sent by NMS |
| Notification | Trap (unreliable), Inform | Sent by Managed Device |
| Response | Response | Response to a message |

Syslog Severity Levels

| 0 | Emergency | Ex: Frozen or force restart the OS |
| --- | --- | --- |
| 1 | Alert | Ex: Failure of database auth system- halts regular operations |
| 2 | Critical | Ex: Hardware component failure |
| 3 | Error | Ex: Auth failures, DNS lookup failures |
| 4 | Warning | Ex: High CPU load, certificate expiration |
| 5 | Notice/Notification | Ex: Interface Up/Down messages, critical config. changes |
| 6 | Informational | Ex: Successful logon or log off events, process completions |
| 7 | Debugging | Ex: System diagnostics or troubleshooting |

Remember: Every Awesome Cisco Engineer Will Need Icecream Daily

NTP Configuration

| Ntp server <ip> | Sets server that device gets its time from, static client/server mode |
| --- | --- |
| Ntp master | Makes the device an NTP server (default stratum of 8), server mode |
| Ntp peer <ip> | Symmetric active mode - peers with other devices |
| Ntp broadcast client | Listens for broadcast messages on that port from NTP servers |

DNS Configuration Commands

| Configure the DNS server the router will send queries to | Ip name-server <ip> |
| --- | --- |
| Configure the domain of the router | Ip domain name <name> |
| Configure the router to act as a DNS server | Ip dns server |
| Enable the router to perform DNS queries | Ip domain lookup |

Static NAT Configuration

| Static NAT | Ip nat inside source static <inside local> <inside global> |
| --- | --- |
| Apply to Interfaces | Ip nat inside, ip nat outside |

Dynamic NAT/PAT Configuration

| Create Pool | Ip nat pool <name> <start ip> <end ip> netmask <mask> |
| --- | --- |
| Create ACL | Access-list <acl number> permit <source network> <wildcard> |
| Config. NAT/PAT | Ip nat inside source list <acl num> pool <pool name> [overload] |
| Apply to Interfaces | Ip nat inside, ip nat outside |

DHCP Messages

| Message | Sent by | Note |
| --- | --- | --- |
| DISCOVER | Client | Broadcast |
| OFFER | Server | Broadcast or Unicast |
| REQUEST | Client | Broadcast |
| ACK | Server | Broadcast or Unicast |
| RELEASE | Client | Unicast, release a DHCP IP address |
| NAK | Server | Server declines a client request |
| DECLINE | Client | Declines a server offered DHCP IP address |

DHCP Snooping Message Checks

| DHCP Messages Received on Untrusted Port | Snooping Checks |
| --- | --- |
| DISCOVER, REQUEST | Source MAC to CHADDR for match |
| RELEASE, DECLINE | Source IP to Interface for match (binding) |

DHCP Snooping Configuration List

| Enable DHCP Snooping Globally, and on VLANs | (config)# ip dhcp snooping (config)# ip dhcp snooping vlan <vlan-ids> |
| --- | --- |
| *For L2 Switches (Access switches): disable Option 82 | (config)# no ip dhcp snooping information option |
| Trust interfaces connected to servers / upstream networking devices | (config-if)# ip dhcp snooping trust |
| Enable Rate Limiting (Best practice) | (config-if)# ip dhcp snooping limit rate <pps> |

DAI Message Checks

Sender MAC to Sender IP in DHCP Snooping Binding Table and/or ARP ACL

DHCP Snooping vs DAI Comparison

| DHCP Snooping | DAI |
| --- | --- |
| Needs 2 Commands to enable (1 global, 1 on a VLAN) (ip dhcp snooping) (ip dhcp snooping vlan [vlan-id]) | Only needs one command to enable (ip arp inspection vlan [vlan-id]) |
| Rate limiting disabled by default | Rate limiting enabled by default (15 pps) |

Port-Security Defaults

After the switchport port-security command on an interface:

Max of 1 MAC address allowed (Dynamically learned)

Violation mode: Shutdown

Static MAC Address Aging Disabled

No Errdisable Recovery

*Remember: Port-Security only allowed on Static Access or Trunk ports, DTP must be off

Port-Security Violation Modes

| Shutdown | Restrict | Protect |
| --- | --- | --- |
| Errdisable Interface if violated | Interface stays up, drops unknown packets | Interface stays up, drops unknown packets |
| Generates SNMP/Syslog messages | Generates SNMP/Syslog messages | Does not generate SNMP/Syslog messages |
| Increments violation counter | Increments violation counter | Does not increment violation counter |

SSH Config List

- Hostname (hostname <name>)

- Domain Name (ip domain-name <name>)

- Generate Key Pair (crypto key generate rsa)

- Username and Password (username <name> secret <password>)

- VTY config (line vty 0 15) (transport input ssh) (login local)

ACL Types

| ACL Type | Numbered Range | Should Be Configured Closest To |
| --- | --- | --- |
| Standard | 1-99, 1300-1999 | Closest to Destination |
| Extended | 100-199, 2000-2699 | Closest to Source |

Apply an ACL

| Apply ACL to VTY Lines | access-class <name/number> [in/out] ← IPv4 ipv6 access-class <name/number> [in/out] ← IPv6 |
| --- | --- |
| Apply ACL to Interface | ip access-group <name/number> [in/out] ← IPv4 ipv6 traffic-filter <name/number> [in/out] ← IPv6 |
| Apply ARP ACL to DAI | ip arp inspection filter <acl name> vlan <vlan num> |

Security

Password Configuration Commands

| Enable secret <password> | Configures and encrypts a clear-text password for privileged exec mode (md5) |
| --- | --- |
| Enable password <password> | Configures a clear-text password for privileged exec mode |
| Service password-encryption | Enables global password encryption |
| Password 7 <hash> | Configures an encrypted vty login password (type 7) |
| Enable secret 5 <hash> | Configures a previously encrypted password for enable mode (md5) (Must plug in a hash, not the actual password) |
| Enable secret level <num> <password> | Configures a password to access a specified privilege level (0-15) |
| Username <name> [privilege (0-15)] {password (encryption-type) <password>} | Configure a user account with a certain privilege level. *Can replace password with secret for md5 encryption |

Password Encryption Levels

| Type 0 (clear-text) | Enable password |
| --- | --- |
| Type 5 (md5) | Enable secret |
| Type 7 (weak) | Service password-encryption |
| Type 9 (scrypt) | User <name> algorithm-type scrypt secret <password> |

Privilege Levels

| 15 | Privileged Exec mode, most commands issued here |
| --- | --- |
| 1 | User Exec mode, default nonprivileged login prompt level |
| 0 | 5 commands default - disable, enable, exit, help, logout |

AAA Authentication Configuration

| Create Model | (config)# aaa new-model |
| --- | --- |
| (As Needed) Define RADIUS Server | (config)# radius-server host <ip> key <password> |
| Create AAA list | (config)# aaa authentication login {default ¦ list-name} <method(s)> |
| Apply to Console/Vty lines | (config-line)# login authentication {default ¦ list-name} |

TACACS+ and RADIUS

| TACACS+ (Cisco) | RADIUS (Industry) |
| --- | --- |
| TCP 49 | UDP 1645, 1812 |
| Encrypts entire packet | Doesn't encrypt entire packet |
| Can authorize a user to a subset of CLI Commands | Does not support subset authorization function |

Random Timers (Defaults)

| OSPF LSA max age | 30 minutes |
| --- | --- |
| Err-disable recovery interval | 5 minutes |
| STP Timers | Hello = 2 seconds, Max Age = 20 seconds Forward Delay = 15 seconds |
| RSTP Timers | Hello = 2 seconds, Keepalive = 3 BPDUs |
| Dynamically learned MAC Addresses | Aging time = 5 minutes |
| HSRP Timers | Hello = 3 seconds, Holdtime = 10 seconds |

QoS Topics

Methods to Classify Traffic

| PCP (also known as CoS) | 802.1Q VLAN tag (Ethernet Header) Only on Trunk links, or Access ports with Voice VLAN (PC traffic not tagged) |
| --- | --- |
| DSCP | Layer 3 QoS (IP Header) |

DSCP Markings

| DF (Default Forwarding) | Best Effort (value of 0) |
| --- | --- |
| EF (Expedited Forwarding) | Usually Voice (value of 46) |
| AF (Assured Forwarding) | AFxy (x=priority, y=drop precedence) |
| CS (Class Selector) | 8 values (backwards compatibility with IPP) |

Assured Forwarding Values

| AF41 (Best Treatment) | AF42 | AF43 |
| --- | --- | --- |
| AF31 | AF32 | AF33 |
| AF21 | AF22 | AF23 |
| AF11 | AF12 | AF13 (Worst Treatment) |

RFC Recommendations

| Voice Traffic | EF (Expedited Forwarding) |
| --- | --- |
| Interactive Video | AF4x |
| Streaming Video | AF3x |
| High Priority | AF2x |
| Best Effort | DF (Default Forwarding) |

PoE Standards

| Name | Standard | Watts | Powered Wire Pairs |
| --- | --- | --- | --- |
| Cisco Inline Power | Cisco | 7 | 2 |
| PoE (type 1) | 802.3af | 15 | 2 |
| PoE+ (type 2) | 802.3at | 30 | 2 |
| UPoE (type 3) | 802.3bt | 60 | 4 |
| UPoE+ (type 4) | 802.3bt | 100 | 4 |

Interactive Audio Recommendations

| One-Way Delay | 150ms or less |
| --- | --- |
| Jitter | 30ms or less |
| Loss | 1% or less |

VoIP Configuration Commands

| Switchport voice vlan dot1p | Separates voice and data traffic, but does not create a separate voice VLAN |
| --- | --- |
| Switchport voice vlan none | Disable the voice VLAN, voice and data traffic unseparated |
| Switchport voice vlan <num> | Sends voice traffic tagged in the configured voice VLAN |
| Switchport voice vlan untagged | Voice traffic goes through the native/default VLAN |

WAN Topologies

Metro Ethernet Topologies

| Name | Topology Type | Note |
| --- | --- | --- |
| E-Line | Point-to-point | Similar to Leased Line |
| E-LAN | Full Mesh | Acts like a LAN |
| E-Tree | Hub and Spoke | Remote sites must go through Central site |

MPLS Types

| MPLS Layer 3 VPN | MPLS Layer 2 VPN |
| --- | --- |
| CE routers form OSPF adjacencies with PE routers | CE routers only form OSPF adjacencies with other CE routers |
| WAN operations, each branch has own IP subnet | WAN is invisible, acts like a giant LAN switch |

VPN Types

| Site-to-site VPN | Remote-Access VPN |
| --- | --- |
| Usually office to office, permanent, multiple devices supported | On demand, one device creates VPN to connect to the enterprise network |
| IPsec encryption | TLS encryption |
| IPsec + GRE for tunnels | Forms tunnels through TLS |
| Does not require additional software | Requires software on end-user devices |
| Network Device to Network Device | User Device to Network Gateway Device |

Wireless Topics

Wireless Standards

| 802.11 | 2.4Ghz | 2 Mbps |  |
| --- | --- | --- | --- |
| 802.11b | 2.4 Ghz | 11 Mbps |  |
| 802.11a | 5 Ghz | 54 Mbps |  |
| 802.11g | 2.4 Ghz | 54 Mbps |  |
| 802.11n | 2.4 / 5 Ghz | 600 Mbps | Wi-Fi 4 |
| 802.11ac | 5 Ghz | 6.93 Gbps | Wi-Fi 5 |
| 802.11ax | 2.4 / 5 / 6 Ghz | 4*802.11ac | Wi-Fi 6 |

Wireless Standards Misc.

| 802.11v | Network-assisted power savings, uses DMS, client can sleep |
| --- | --- |
| 802.11r | Fast transition roaming, perform PMK calculation in advance |
| 802.11w | Management Frame Protection, security mechanism |
| 802.11k | Assisted roaming, Client requests a list of candidate transition APs |

802.11 Frame Order

802.11 Message Types

| Management | Control | Data |
| --- | --- | --- |
| Beacon | RTS (Request to Send) | Used to send data packets |
| Probe request, response | CTS (Clear to Send) |  |
| Authentication | ACK |  |
| Association request, response |  |  |

RF Terms

| Amplitude | Height of a wave from trough to crest |
| --- | --- |
| Wavelength | Length of a wave from crest to crest |
| Frequency | Number of waves per given time period |
| Absorption | Wireless signal passes through material, weakening signal |
| Reflection | Signal bounces off a material, ex. metal |
| Refraction | Signal goes through a medium, changes direction and speed |
| Diffraction | Wave encounters object and travels around it, can cause blind spots |
| Scattering | Material causes signal to scatter in all directions, ex. dust, smog |

Service Sets

| IBSS | Clients connect directly, no AP needed (ad hoc) |
| --- | --- |
| BSS | Clients connect to a single AP |
| ESS | Multiple APs connect via wired network, each BSS has same SSID, different BSSIDs, different channels |
| MBSS | APs connect to each other wirelessly, one channel for AP connection, one channel for the BSS |

AP Overlap

Acceptable overlap for consistent roaming = 10-15%

Autonomous AP Modes

| Repeater | Extend range of BSS |
| --- | --- |
| WGB (Workgroup Bridge) | Wireless client of another AP, connect wired client to wireless network |
| Outdoor Bridge | Long distance, point-to-point |

Lightweight AP Modes

| Local | Default, sends all client traffic for all WLANs in CAPWAP to the WLC |
| --- | --- |
| FlexConnect | Local, but can act Autonomous if connection to WLC goes down |
| Sniffer | No BSS offer, sends traffic to Wireshark or similar |
| Monitor | No BSS, to detect and disassociate rogue devices |
| Rogue Detector | No radio, detects rogue devices on the wired network |
| SE-Connect | No BSS, dedicated to RF spectrum analysis |
| Bridge | Bridge for long distance, can form mesh |
| Flex Plus Bridge | FlexConnect + Bridge mode |

WLAN Architectures

| Autonomous | Autonomous AP, configured manually, all logic on the AP, connects via trunk port, |
| --- | --- |
| Lightweight | Split-MAC, AP handles lightweight functions, WLC does everything else, Lightweight AP connects via access port |
| Cloud-Based | Between Split-MAC and Autonomous, can configure itself, only management/control traffic is sent to the WLC |

WLC Deployments

| Unified (Hardware in central location) | Max 6000 APs |
| --- | --- |
| Cloud-Based (WLC is a VM on a server) | Max 3000 APs |
| Embedded (WLC in a switch) | Max 200 APs |
| Mobility Express (WLC in an AP) | Max 100 APs |

WLC Ports (Physical)

| Service Port | Dedicated management port, must connect to access port (one VLAN) |
| --- | --- |
| Distribution System Port | Standard network port, wireless to wired network, can form a LAG |
| Console Port | Console, RJ45 or USB |
| Redundancy Port | Connect to another WLC - form HA (High Availability) pair |

WLC Interfaces (Logical)

| Management | Management traffic (Telnet, SSH, HTTP(S), RADIUS, CAPWAP) |
| --- | --- |
| Redundancy Management | To connect and manage the “standby” WLC in the HA pair |
| AP-Manager | Manage the lightweight APs, source IP for the lightweight APs |
| Virtual | Communicate with wireless clients - relay DHCP requests, etc. |
| Service Port | Bound to physical service port, out-of-band management |
| Dynamic | Map WLAN to VLAN, send wireless to the wired LAN |

CAPWAP Tunnels

| Control (UDP 5246) | Data (UDP 5247) |
| --- | --- |
| Configure APs, control/management operations | Traffic from wireless clients sent through this tunnel |
| Encrypted by default | Not encrypted by default |

Encryption and Integrity Protocols

| Protocol | Encryption | Integrity | Used In |
| --- | --- | --- | --- |
| TKIP | RC4 | MIC | WPA |
| CCMP | AES Counter Mode | CBC-MAC | WPA2 |
| GCMP | AES Counter Mode | GMAC | WPA3 |

WPA2/3 Modes

| WPA2 Personal | PSK Authentication |
| --- | --- |
| WPA2 Enterprise | 802.1x + EAP Authentication |
| WPA3 Personal | SAE to replace PSK handshake (protection against dictionary attacks) + PMF (Protected Management Frames) |
| WPA3 Enterprise | 802.1x + EAP Authentication, Mandatory PMF |

Wireless Authentication

| Open Authentication | Client sends authentication request, no questions asked |
| --- | --- |
| WEP | Auth and encryption (RC4), shared key protocol, AP sends challenge phrase |
| LEAP | Client provides User and Pass, mutual auth via challenge phrases |
| EAP-FAST | PAC from server to client, TLS tunnel created, then client auth happens in TLS tunnel |
| PEAP | Digital certificate to auth the server and establish TLS tunnel, then client auth inside of TLS tunnel |
| EAP-TLS | Requires a certificate on the AS and every supplicant, TLS tunnel used to exchange encryption key information |

WLAN GUI Security

| VPN Pass-through | Layer 3 | Security for WLANs |
| --- | --- | --- |
| Web Authentication | Layer 3 | Security for Guest LANs |
| Web Passthrough | Layer 3 | Access for Guest LANs |
| 802.1x | Layer 2 | EAP + Dynamic WEP Key |
| None + EAP Passthrough | Layer 2 | Open auth + remote EAP auth |
| WPA + WPA2 | Layer 2 | WPA or WPA2 |
| 802.1x + CCKM | Layer 2 | 802.1x clients won’t need to reauthenticate when roaming |

WLC GUI Tabs

| Monitor | WLANs | Controller | Wireless | Security | Management |
| --- | --- | --- | --- | --- | --- |
| See AP and Client Summary | Create/edit WLANs, (general, security, QoS, Advanced) | Create/edit interfaces, manage ports | View APs, edit AP modes, radio config., QoS config. | Create/apply CPU ACLs, View AAA settings | View Management Settings, config. management connections |

WLAN GUI QoS Settings

Platinum (voice)

Gold (video)

Silver (best effort) (default)

Bronze (background)

WLC CLI Configuration Commands

| Allow HTTP connections (disabled by default) | config network webmode {enable ¦ disable} |
| --- | --- |
| Allow HTTPS connections (enabled by default) | config network secureweb {enable ¦ disable} |
| Enable/disable Telnet connections | config network telnet{enable ¦ disable} |
| Enable/disable SSH connections | config network ssh {enable ¦ disable} |
| Config. minutes of inactivity before timeout from telnet/SSH sessions (0 = no timeout) | config sessions timeout <timeout> |
| Config. number of simultaneous Telnet/SSH sessions allowed | config sessions maxsessions <session_num> |

Virtualization

VMs and Containers

| Virtual Machines | Containers |
| --- | --- |
| Require a Hypervisor | Do not require a Hypervisor (shares a kernel) |
| Has dedicated resources | Share host resources |
| Requires more overhead | Requires less overhead |

Automation and SDN

SD-Access Deployments

| Greenfield (Optimal) | Brownfield |
| --- | --- |
| Entirely new deployment | Uses existing network |
| DNA Center configures the underlay | Underlay not configured by DNA Center |
| All switches and links are Layer 3 | Some switches might be Layer 2 |
| IS-IS used as routing protocol | Any routing protocol (recommended IS-IS) |
| No need for STP | Layer 2 switches still need STP |
| No need for a FHRP | Layer 3 switches use a FHRP |
| Access layer switches are default gateway | Distribution switches are default gateway |

SD-Access Protocols

| Cisco TrustSec | Provides policy control |
| --- | --- |
| LISP | Provides the control plane |
| VXLAN | Provides the data plane |

SD-Access Layers

| Application Layer | Contains scripts, applications; talks to the Northbound API |
| --- | --- |
| Control Layer | Contains the Controller (DNA Center, now called Catalyst Center) |
| Infrastructure Layer | Contains the network devices that forward messages on the network |

SD-Access Architecture

| Underlay | The physical network of devices and connections, switches use a separate IP address space than the rest of the enterprise |
| --- | --- |
| Overlay | The virtual network built on top of the underlay, uses VXLAN |
| Fabric | Combination of overlay and underlay, the network as a whole |

SD-Access Underlay Switch Roles

| Edge Node | Connects to end hosts |
| --- | --- |
| Border Node | Connects to devices outside SD-Access domain, ex: WAN routers |
| Control Node | Uses LISP for control plane functions |

CRUD to HTTP Messages

| CRUD Acronym | HTTP Message Equivalent | Example |
| --- | --- | --- |
| Create | POST | Generate + retrieve token |
| Read | GET | Retrieve data from object |
| Update | PUT, PATCH | Add/modify object |
| Delete | DELETE | Remove an object |

HTTP Response Codes

| Class of Response | Class Name | Example |
| --- | --- | --- |
| 1xx | Informational | 102 - Processing |
| 2xx | Successful | 200 - OK, 201 - Created, 202 - Accepted |
| 3xx | Redirection | 301 - Moved Permanently 302 - Moved Temporarily |
| 4xx | Client Error | 401 - Unauthorized (auth fail) 404 - Not Found |
| 5xx | Server Error | 500 - Internal Server Error |

Data Serialization Languages

| JSON | XML | YAML |
| --- | --- | --- |
| REST API Usage | REST API Usage | REST API Usage (not often) |
| Whitespace Insignificant | Whitespace Insignificant | Whitespace Significant |
| Key:value | <key>value</key> | Key:value |
| “Interface f/01”:{“status”:”up”} | <int>G0/1</int> | - interface: G0/1 |

REST Authentication

| Basic Authentication | Username + Password in HTTP Authorization Header |
| --- | --- |
| Bearer Authentication | Token in HTTP Authorization Header - Tokens expire |
| API Key Authentication | Static Key issued by API - valid until revoked |
| OAuth 2.0 | Grants third parties limited access to resources - access delegation Ex: logging in with Google |

Ansible and Terraform

| Ansible | Terraform |
| --- | --- |
| Agentless | Agentless |
| Push Model | Push Model |
| Configuration Management Tool | Infrastructure Provisioning Tool |
| Procedural | Declarative |
| Python (Playbooks written in YAML) | Core written in GO, Files written in HCL |
| Red Hat (Open source) | HashiCorp |

Miscellaneous

Hardware

| CPU | Management and Control plane operations |
| --- | --- |
| ASIC | Data plane operation, processes data traffic for forwarding |
| TCAM | Stores tables, ACLs, QoS lookups, returns entry to ASIC for forwarding |

Formulas

| Conversion | Formula | Example |
| --- | --- | --- |
| AF to DSCP | 8x + 2y (x = Priority, y = Drop Precedence) | AF32 = 8(3) + 2(2) = DSCP 28 |
| CS to DSCP | 8(CS value) | CS2 = 8(2) = DSCP 16 |
| Prefix to Usable Hosts (IPv4) | 2h-2 (h = host bits) | /24 = 8 host bits, 28-2 = 254 usable hosts |
| Prefix to Usable Hosts (IPv6) | 2h (h = host bits) | /120 = 8 host bits 28= 256 usable hosts *IPv6 does not reserve first or last addresses (network and broadcast) |
| OSPF Interface Cost | Reference Bandwidth / Interface Bandwidth = Cost | Default Reference = 100Mbps 100/1000(GigEthernet) = 1 *Lowest cost can only be 1 |
| EIGRP Metric | Bandwidth (slowest link) + Delay (all links) = Metric What really happens: [(10,000,000 / bandwidth in kbps) + (total delay in usec / 10)] x 256 | 100 Mbps = 100,000 Kbps Total Delay = 400 usec 10,000,000/100,000 = 100 400/10 = 40 So: 256 × (100+40)=35,840 Metric |

References

CCNA Exam Topics https://learningnetwork.cisco.com/s/ccna-exam-topics

Jeremy’s IT Lab Playlist https://youtu.be/H8W9oMNSuwo

Cisco Wireless Controller Configuration https://www.cisco.com/c/en/us/td/docs/wireless/controller/8-5/config-guide/b_cg85/administration_of_cisco_wlc.html#ID562

Boson ExSim https://boson.com/exsim-max-practice-exams/